ResoScan

Non-Invasive Biomechanical Diagnostics via Resonant Modal Spectroscopy

Mentor Briefing Document  —  Unisys Innovation Program 2026

Ramaiah Institute of Technology, Bengaluru

# Team & Mentor Details

## Student Team

| Name | Department | Contact |
| --- | --- | --- |
| Yashas N | Artificial Intelligence & Data Science | yashraaj2005@gmail.com |
| Jeeth Bhavesh Kataria | Artificial Intelligence & Data Science | jeethkataria9798@icloud.com |
| Naveen Gopalakrishna Patil | CSE (AI & Machine Learning) | naveengpatil26@gmail.com |
| Aditya Sarap | Electronics & Telecommunication | adityasarapworld@gmail.com |

## Project Guide

Dr. Sowmya B. J., Associate Professor, Dept. of Artificial Intelligence & Data Science, M S Ramaiah Institute of Technology | sowmyabj@msrit.edu

# Executive Summary

ResoScan is a low-cost, portable, handheld medical device that performs non-invasive biomechanical tissue diagnostics. It digitizes the centuries-old clinical technique of percussion and palpation, turning a physician's subjective tap-and-feel into quantifiable, objective data. Target Cost: Under ₹8,000 BOM ¦ Applications: Orthopedics, Pulmonology, Urology ¦ Technology: Resonant Modal Spectroscopy + ML

# Problem Statement & Motivation

## Why Does This Problem Exist?

Modern medical diagnostics relies on imaging modalities like X-ray, CT, MRI, and Ultrasound. While powerful, these share a critical limitation: they reveal structure, not function. For example:

- A healed fracture may appear bridged on X-ray weeks before it can safely bear weight.

- A mass detected on ultrasound cannot be characterized without additional invasive testing.

- These technologies require expensive infrastructure, trained radiologists, and often expose patients to ionizing radiation.

## The Five Core Problems

| Problem | Description |
| --- | --- |
| Structure-Function Gap | Radiographic healing lags behind mechanical strength recovery, causing premature loading or unnecessary immobilization. |
| Subjectivity of Clinical Exam | Percussion and palpation depend on clinician skill; reported sensitivities as low as 56% for certain conditions. |
| Cost Inaccessibility | FibroScan costs ₹16-37 Lakhs, MyotonPRO ₹5-6 Lakhs, Osstell ₹3-4 Lakhs — far beyond most clinics. |
| Invasive Monitoring | Bladder compliance, fracture stability, and lung pathology often require invasive or radiation-heavy procedures. |
| The 'Missing Middle' | No intermediate functional diagnostic device exists between ₹1,000 clinical tools and ₹15+ Lakh imaging systems. |

# Proposed Solution: How ResoScan Works

ResoScan employs Resonant Modal Spectroscopy (RMS) — a technique that delivers controlled mechanical impulses to tissue and analyzes the vibrational response to characterize tissue type and health state.

## Core Operating Principle

- A Voice Coil Actuator (VCA) delivers controlled mechanical impulses across a programmable frequency range of 20 Hz to 1 kHz.

- A high-bandwidth MEMS accelerometer captures the tissue's vibrational response.

- Fast Fourier Transform (FFT) analysis extracts resonant frequencies, damping characteristics, and spectral signatures.

- Machine learning algorithms classify tissue state: fractured vs. healed bone, air-filled vs. consolidated lung, compliant vs. stiff bladder.

## Signal Processing Pipeline (5 Stages)

| Stage | Description |
| --- | --- |
| 1. Excitation Generation | ESP32 DAC generates chirp (20-1000 Hz sweep), impulse (Gaussian pulse), or single-frequency waveforms. |
| 2. Response Acquisition | ADXL343 accelerometer samples at 3200 Hz via I2C; streams to PC via USB Serial at 115200 baud. |
| 3. Spectral Analysis | Windowed FFT (Hanning, 1024 points), Power Spectral Density (PSD), peak detection, damping ratio estimation. |
| 4. Feature Extraction | Extracts resonant frequency, amplitude, Q-factor, damping ratio, and spectral centroid. |
| 5. ML Classification | Random Forest / SVM classifier outputs Normal/Abnormal with a confidence score for clinical interpretation. |

FIG 1. End-to-End Signal Flow & Analysis Pipeline for ResoScan

# Clinical Applications

ResoScan targets three initial clinical domains with well-defined, measurable protocols:

## Application 1: Fracture Healing Monitoring (Orthopedics)

- Configuration: Pitch-catch — excitation at medial malleolus (ankle), sensing at tibial tuberosity.

- Key Metric: Tibial Stiffness Index (TSI) = f²_injured / f²_healthy × 100%.

- Clinical Threshold: TSI > 80% indicates the bone is safe for weight-bearing.

- Clinical Value: Removes reliance on lagging X-ray evidence for return-to-load decisions.

## Application 2: Pneumothorax Detection (Pulmonology)

- Configuration: Direct percussion on chest wall, intercostal space, mid-clavicular line.

- Key Metric: Power ratio in 200–400 Hz band (left side vs. right side comparison).

- Clinical Threshold: Ratio > 2.0 indicates hyper-resonance (air trapping = pneumothorax).

- Clinical Value: Enables point-of-care diagnosis by paramedics within minutes — no imaging required.

## Application 3: Bladder Compliance (Urology)

- Configuration: Lamb wave propagation from suprapubic region (20–100 Hz) with lateral sensing.

- Key Metric: Wave velocity cₑ (m/s) measured 5–10 cm from excitation point.

- Clinical Threshold: cₑ > 4 m/s indicates elevated intravesical pressure.

- Clinical Value: Eliminates need for catheterization in routine bladder compliance monitoring.

FIG 3. System Block Diagram for ResoScan

# Hardware Architecture & Bill of Materials

The entire device is designed to be manufactured with a BOM under ₹8,000 INR using readily available components:

| Component | Role | Est. Cost (INR) |
| --- | --- | --- |
| ESP32 MCU | Signal generation (DAC) + data processing + USB communication | ₹415 |
| MEMS Accelerometer (ADXL343) | Capture tissue vibrational response at 3200 Hz | ₹415–665 |
| Voice Coil Actuator (VCA) | Deliver controlled mechanical impulses to tissue | ₹1,250–2,075 |
| Amplifier | Drive the VCA with sufficient power | ₹165–415 |
| Force Sensor | Standardize probe contact force for repeatability | ₹250–415 |
| Battery & Charger | Portable power supply | ₹415 |
| Enclosure | Handheld ergonomic housing | ₹830 |
| Coupling Tip | Interface between device and body surface | ₹165 |
| Miscellaneous | PCB, wiring, connectors | ₹830 |
| TOTAL BOM | — | ₹5,000–6,500 |

FIG 2. Hardware Schematic for Resonant Modal Spectroscopy Device

# Software Stack

| Layer | Technology / Tools |
| --- | --- |
| Embedded Firmware | Arduino / ESP-IDF on ESP32 |
| Signal Processing | Python (NumPy, SciPy) — FFT, PSD, peak detection |
| Data Visualization | PyQtGraph — real-time spectrogram display |
| Machine Learning | Scikit-learn / TensorFlow — Random Forest, SVM classifiers |
| User Interface (Prototype) | PyQt5 |
| User Interface (Future) | React-based web dashboard |
| Data Storage | SQLite (local) / Firebase (cloud) |

# Innovation & Novelty

What Makes ResoScan Different

Existing commercial devices like Osstell, MyotonPRO, and FibroScan are domain-specific and expensive (₹3-37 Lakhs).

ResoScan generalizes Resonant Modal Spectroscopy across multiple domains in one affordable handheld platform.

It transforms a subjective, skill-dependent clinical technique (percussion) into an objective, reproducible measurement.

At under ₹8,000, it fills the 'Missing Middle' between basic examination tools and expensive imaging systems.

# Expected Outcomes & Deliverables

| Deliverable | Success Criteria |
| --- | --- |
| Functional Prototype | Working handheld device with real-time FFT visualization on PC |
| Material Differentiation Proof | Measurable spectral differences between materials of varying stiffness |
| Biological Signal Acquisition | Successful resonance capture from human tissue (shin bone vs. calf muscle) |
| Stiffness Correlation | Established relationship: resonant frequency² ∝ stiffness / mass |
| ML Classification Accuracy | Greater than 80% accuracy on validation dataset |
| User Interface Dashboard | Real-time spectrogram + stiffness index + clinical indicators |
| Reproducible Protocol | Documented procedure with calibration routines for consistent measurements |
| Cost Validation | Confirmed BOM under ₹8,000 INR |
| Clinical Trial Foundation | Technical documentation suitable for future IRB-approved trials |

# Future Development Roadmap

- Clinical Validation: IRB-approved trials to build clinical evidence.

- Regulatory Pathway: Target FDA Class II / De Novo clearance.

- Miniaturization: Integrate all components into a single handheld unit.

- ML Expansion: Expand classifier models with large clinical datasets.

- Condition-Specific Algorithms: Develop targeted diagnostic modules per condition.

- Use Cases Enabled: Paramedic pneumothorax detection, sports medicine clearance, home bladder monitoring.

# Vision Statement

"ResoScan envisions a future where every clinic, ambulance, and home has access to functional diagnostic technology. By bridging the gap between the art of medicine and the science of measurement, ResoScan aims to democratize diagnostics and improve patient outcomes globally."

# Key References

The project is grounded in published peer-reviewed research:

- Vibrational method for bone fracture healing assessment. PMC, 2021.

- Non-invasive bladder compliance assessment via ultrasound. PubMed, 2021.

- Tabla: Auscultatory percussion device for pneumonia detection. PMC, 2018.

- Lamb wave-based internal pressure measurement. PMC, 2022.

- FDA Clearance of OsteoProbe for bone material strength (K250216). BioSpace, 2025.

- MyotonPRO technology and FibroScan transient elastography methodology.

# Keywords

Resonant Modal Spectroscopy • Non-Invasive Diagnostics • Tissue Stiffness • Mechanical Impedance Analysis • Fracture Healing Monitoring • Pneumothorax Detection • Bladder Compliance • Voice Coil Actuator • MEMS Accelerometer • Fast Fourier Transform • Point-of-Care Diagnostics • Machine Learning Classification • ESP32 • Low-Cost Medical Devices • Portable Healthcare Technology • Lamb Wave Analysis • Digital Percussion • Elastography